Plant Protection Equipment for Chemical Application on Crop to Effectively Control Pests and Diseases

D. M. Kadam1, R.K. Panse2, Prveeen Kalluri3 and S.K. Rajak4

1 Assistant Professor, Department of FMPE, CAE, JNKVV, Jabalpur-482004 (M.P.)

2 Assistant Professor, College of Agriculture Balaghat, JNKVV, Jabalpur-482004 (M.P.)

3 Ph. D. Scholar, Department of FMPE, JNKVV, Jabalpur-482004 (M.P.)

4 Assistant Professor, College of Agriculture Balaghat, JNKVV, Jabalpur-482004 (M.P.)

Corresponding Author: dmkadam.jnkvv@gmail.com

- Abstract

Pesticides play an integral role in the control of pests such as insects, fungi and weeds on field crop and orchards, to produce and conserve food grains, fruits, vegetables and fibre. All insects are not pests. Some help in pollinating plants or feeding on other insects which are pests. Control of pests is a need to protect crop for better yield. Proper use of pesticide is however a critical. Excessive dosage may damage or even kill the plant, whereas under dosage will not give effective control. Desirable flora and fauna and environment may be harmed by indiscriminate use of pesticides. Success of plant protection will depend upon the use of right pesticide, at the right time, with the right technique and by the right equipment. So, an attempt has been made in this chapter to makes aware and familiar farmers, researchers, scientists and readers, with the plant protection equipment, their basic classification,  basic components of sprayers, nozzle and their types, brief description of various types of sprayers and dusters used for plant protection, selection of plant protection equipment, calibration of plant spraying equipment, maintenance of plant protection equipment, safety precautions to be taken by operator before, during and after spraying etc. This crucial information on plant protection equipment will be useful for effectively controlling pests and diseases on field crops as well as orchards.

- Introduction

Chemical application is an important aspect of the modern agriculture to save crop beset by insects, fungus, virus and other parasites, as well as weeds, which are not favourable to cultivated plant growth. Weeds can be eradicated by effective cultivation, but pests and diseases have to be kept under control with chemical spray and powder application. Chemicals are now being used to kill weeds without damaging the crop in addition to the weeding tools. Chemicals are either sprayed in liquid form or applied as a dry powder. Sprayers and dusters are available in many forms for this purpose. Farmers often tend to use large quantity of pesticides than actually required. This causes higher loss of pesticide (nearly one-third of pesticides applied to crop on ground as well as drift), this requires more time for its application and adds to health hazards, ecological and environmental destruction. Therefore, application technique of pesticide should be such that the required quantity of pesticides is applied to cover the target-leaf-foliage with less environment pollution and maximum pest control. Efficient spraying system can ensure more effective spraying resulting in saving of chemical and reduction in losses to environment or carbon foot print in site-specific chemical application in an emerging area in developed world, wherein application of chemical is concentrated at affected part of the field rather than whole field approach currently followed by farmers. This saves substantial amount of chemical along with checking the environmental issues.

- Equipment for Application of Chemicals

The equipment for chemical spraying can be broadly classified based on form of chemical being sprayed. Equipment used for spraying of powdery material is called as dusters and equipment being used for liquid spraying is called as sprayers. In general plant protection equipment can be classified in five categories:

1. Duster

2. Sprayer

3. Fog or Smoke generator

4. Aerial Application (Fixed wing or rotary wing aircraft)

5. Others (seed dressing drum, flame thrower, bird scarer etc.)

3.1 DUSTERS

Duster is a machine to apply chemicals in dust form. Dusters make use of air streams to carry pesticides in finely divided dry form on the plants. A duster essentially consists of hopper, agitator, feed control, fan or blower and delivery tube and nozzle. There are different types of dusters such as plunger type; knapsack type, rotary type and power operated dusters.

Fig. 1. Hand operated duster

3.1.1 Power operated dusters

Field dusters or power operated duster are more common in use, it is mounted upon a platform bolted to the rear of a tractor and operated by PTO. It mainly consists of a power driven fan, a hopper and a delivery spout. The fan creates strong air, which causes the dust to blow off from hopper to a considerable distance either vertically or horizontally. A movable delivery spout suitably fitted with the unit regulates the direction of dust. An agitator is provided in the bottom of hopper to keep the dust in fluffy condition to meter out uniformly through the feed system into the air stream. This type of dusters is used for large areas.

3.1.2 Electrostatic Duster

The primary objective of charging dust particles is to increase the percentage deposition on plant surface. The electrostatic force generally has no greater effect on the large particles and it does not affect basic trajectory from application equipment to the target. But if charged particle reaches the plant, the probability of their deposition increases. Charging dust particles improves the control of insects and pests. The increased deposition efficiency, particularly for small particles, would reduce the amount of drift. Electrostatic dusters and sprayers are more complicated than conventional equipment.

3.2 SPRAYERS

A sprayer is an equipment to atomize the spray fluids into small droplets (to split a liquid into droplets), or create an emulsion (a mixture in which one liquid is suspended as minute globules in another liquid) or a solution and eject it with some force for proper distribution over sprayed surface.

Sprayers can be classified based on different criteria

- Classification based on source of energy and droplet spectra produced

- Classification based on volume of liquid applied

- Classification based on power source utilized

3.2.1 Classification based on source of energy and droplet spectra produced

Based on the source of energy sprayers are classified in to three groups i.e. hydraulic energy sprayer, gaseous energy sprayer and centrifugal energy sprayer. The broad classification is given in Table 1.

Table 1.  Pesticide sprayers may be classified on the basis of source of energy and     droplet spectra produced

| According to source of energy |  |
| --- | --- |
| Hydraulic energy sprayer | Syringes, hand sprayers, telescopic lance sprayers, stirrup pump sprayers, lever operated knapsack sprayers, rocking sprayers, foot sprayers, hand compression sprayers, power sprayers, tractor operated sprayers |
| Gaseous energy sprayer | Hydraulic energy sprayers |
| Centrifugal energy sprayer | Spinning disc sprayers |
| According to droplet spectra produced |  |
| Less than 50 micron | Rotary atomizers, thermal foggers, aerosol generators |
| 50-100 micron | Gaseous (mist blowers) and centrifugal energy sprayers (ULV sprayers) |
| 100-150 micron | Gaseous / centrifugal energy sprayer (rotary atomizer) and dusters |
| Greater than 200 micron | Hydraulic energy sprayers |

Most sprayers in use today are hydraulic types in which spray pressure is built up by the direct action of pump on liquid spray material. The pressure thus developed forces the liquid through nozzles, which break the liquid into fine size droplets and disperse them in the desired spray pattern. Also sufficient energy is imparted to the spray droplets to carry them from nozzle to the surface to be treated. The main parts of hydraulic sprayer are; pump, tank containing an agitator, framework for mounting sprayer, pressure regulator and relief valve, pressure gauge, strainers and screens, control valves, pipes and fittings, distribution system and power source.

3.2.2 Classification based on volume of liquid applied

Sprayers can be classifie d into three groups i.e., High volume sprayer, low volume sprayer and ultra-low volume sprayer. High volume sprayers are those where more than 400 litres of spray liquid per hectare is to be sprayed. If spray volume is in between 5 to 400 litres per hectare, these sprayers are called as low volume sprayers. Sprayers applying less than 5 litres per hectare are called as Ultra-low volume sprayers.

3.2.3 Classification based on power source used

Sprayers are of four types viz. manually operated, animal drawn, power tiller or tractor drawn and self-propelled sprayers.

3.2.3.1 Manually Operated Sprayer

3.2.3.1.1 Stirrup-Pump Sprayer: It consists of a brass pump, a footrest (stirrup), hose, lance and nozzle. It may have a single barrel or double barrel pump. In this type of sprayer continuous pumping is necessary for getting uninterrupted discharge of spray fluid. It develops a pressure of 4-10 kg/cm. It is suitable for small scale field spraying. It does not give uniform spray, as single barrel pump has no mechanism to retain pressure. Two persons are required for operating this sprayer; one to operate the pump and other to direct spray from the nozzle at the end of spray lance. The length of nozzle varies depending upon the requirement in the field but it is generally kept as 5 m.

Fig. 2. Stirrup-Pump Sprayer

3.2.3.1.2 Hand compression low volume sprayer

This type of sprayer consists of a tank and a plunger pump operated by hand. The tank is made of brass alloy sheet with capacity of 9 to 14 liter depending upon the model. It is suitable for controlling pests and diseases on tea, coffee, cotton, potatoes, paddy, jute, sugarcane and vegetables. This sprayer is suitable for controlling pests, diseases on tea, coffee, cotton, potatoes, paddy, jute, sugarcane and vegetables.

Fig. 3. Hand compression sprayer

3.2.3.1.3 Lever operated knapsack (LOK) sprayer: The major components of LOK sprayer are tank, cylinder, piston, pressure chamber, handle, lance and nozzles which need to be designed or selected depending on application requirement. In a LOK sprayer with liquid filled in the tank, initially the pressure chamber is occupied by air at atmospheric pressure. When the handle moves upwards, the piston also moves upwards resulting in suction stroke and a certain volume of liquid enters into the cylinder. During the pressure stroke the handle moves downwards causing downward movement of the piston and the liquid in the pump cylinder is forced through a valve into the pressure chamber. This results in trapping of air in the upper portion of the pressure chamber (trigger in closed position). Thus, in each stroke certain volume of liquid is forced into the pressure chamber reducing the volume of air and at the same time increasing the pressure inside it.

Fig. 4. Lever operated knapsack (LOK) sprayer

3.2.3.1.4 Foot-Operated Sprayer: It is also called pedal pump. It consists of a plunger assembly, stand, suction hose, delivery hose and an extension rod with nozzle. One end of suction hose is fitted with a strainer and other end with a flexible coupling. Similarly, one end of delivery hose is fitted with a cut-off valve and other end with a flexible coupling. An additional container is required to hold spray fluid, as this sprayer does not have a built-in tank. Continuous pedalling is required for uniform spray. It can develop a pressure of 17-21 kg/cm. It is easy to operate and can be used for spraying tall crops and fruit trees up to 4 m height. Sprayer can be used to spray trees height up to 6 m with additional hose.

Fig. 5. Foot-Operated Sprayer

3.2.3.1.5 Rocking Sprayer: It consists of a pump assembly, platform, operating lever, pressure chamber, and suction hose with strainer, delivery hose and an extension rod with spray nozzle. Rocking movement of handle operates the pump resulting in built up pressure in pressure chamber. This also requires an additional container to hold fluid. It can develop a pressure of 14-18 kg/cm, but in some cases it can develop up to 36 kg/cm. It can be used for spraying coconut and palm trees.

Fig. 6. Rocking Sprayer

3.2.3.1.6 Motorized Knapsack Sprayer: This is also called power operated gaseous-energy knapsack sprayer. The hopper having capacity of 7-12 litres, is made of high density polyethylene. Another small tank of 0.75-2.25 litres capacity is also provided for fuel. It is powered by 1-3 hp engines. The fuel consumption varies from 0.6-2.0 l/ha. The frame is provided with shock- proof cushion that comfortably fits on the back of operator and eliminates the transfer of vibrations of the engine to the operator. It is provided with an on-off control switch and a mechanism for adjusting the discharge rate from 0.5-5.0 l/min. The spray liquid is blown- out by an air current generated in the tank. A part of air generated is diverted into the hopper to form an air cushion over the liquid in the tank. This ensures a uniform delivery of liquid. The liquid from tank passes through a tube to the nozzle on the spray lance by gravity. Effective swath width is 4-5 m horizontally and 3-4 m vertically. It can cover approximately 3 ha a day. The machine fitted with a rotary pump and tree-spray lance can spray trees up to 6 m high. Some of these sprayers can be converted for dusting and ULV application.

Fig. 7. Motorized Knapsack Sprayer

3.2.3.1.6 Hydro-pneumatic Sprayers: It has the same range of use as the low-pressure, low- volume sprayer. The spray liquid is carried in a pressure tank and spraying pressure is developed by means of an engine-powered air compressor. The spray material therefore does not pass through the pump or contact any other moving parts. The sprayers are available in mounted types and tank size is limited to about 1000 litres because of cost and weight of pressure tank. Agitation is provided either by means of a mechanical agitator or by an air tube discharging air below the surface of liquid in the tank.

3.2.3.1.7 Electrodynamic Sprayer: In recent years increasing interests have been using less solvent spraying pesticides. There have been many developments in this direction and development of sprayer based on charging of droplets is one of them. Operation of an electro-dynamic sprayer is based on the droplets emerging from the delivery gun with an electric charge. Since the droplets have the same charge, droplets repel each other causing them to form a reasonably wide spray. Electro-dynamic spraying uses rotary atomization principle. The principal feature of electrodyn sprayer is the device for providing a high- tension voltage in a very small space through which the liquid emerges by gravity. The source of power in this sprayer is four standard torch light batteries that last for about 60 hours. Since there are no moving parts, the sprayer has higher life span. The sprayer has a backpack unit enclosing the charging circuit and battery and a long handle having multiplier unit. The spray tank and nozzle are supported at the other end of handle. A high voltage of 20 kV is applied to the stud inside the nozzle, which creates a powerful field against the ground electrode outside the nozzle. This field pulls out the liquid from the tank and disintegrates it.

3.2.3.1.8 Ultra-Low Volume Sprayer (ULV): The equipment use high rpm spinning disc devices. Motorized knapsack sprayers and aircraft fitted with special spinning disc attachment are also used as ULV sprayers. ULV sprayers use blower for creating a blast of air for atomizing the spray liquid and for carrying the droplets to the target. The low volume concentration (LVC) used in ULV perform better due to higher initial deposits. LVC droplets remain in the liquid form and more readily picked up by the pest that gives higher mortality of the pests. The sprayer has a motor powered by 6 to 12 volt batteries. The spinning disc having grooves or teeth is attached to motor and rotates at 4000-9000 rpm. Average droplet size varies from 35-100 micron.

Fig. 8. Spinning disc ULV Sprayer

3.2.3.2 Bullock Drawn Sprayer: The bullock drawn sprayer is used for spraying of pesticides on field crops such as soybean, groundnut, gram cotton etc. A pair of bullocks is used to operate the two reciprocating pumps. It is mounted with a tank of 200 litre capacity. The pressure produced by the pump is used to spray the chemicals by a boom having six hollow cone nozzles. The average discharge is 440 ml/min. The draft and the field capacity of the sprayer are 900N and 0.56ha/h when operated at the forward speed of 2.4 km/h.

Fig. 9. Bullock drawn boom sprayer

3.2.3.3 Tractor / Power tiller drawn sprayer: Power sprayers are operated usually with internal combustion engines of 1-5-hp capacities or by a tractor. A small engine operated power unit ensuring a constant steady pressure operates the pressure pump. They are operated at pressure from 20-55 kg/cm. The pump is operated by tractor PTO. These sprayer consists of a pump, one or more drums, control valves, pressure gauge, pressure regulator, relief valve and a spray boom fitted with nozzles. The boom may be of flexible hosepipe on which nozzles are mounted to meet different row spacing. The boom is tied with a rigid beam by clamps. Inlet liquid supply to boom is provided at two points for even distribution of liquid. Holes are provided on the frame to lower or raise the beam to suit the crop height. Normally sprayer boom has 13-15 triple action nozzles and can cover 7-8 m width. The capacity of tractor- mounted sprayer is about one hectare per hour while operating at a speed of 3.5 km/h. The tractor is operated in the same pathways and in the same direction for all sprays. Sprayers are used in row and tall crops such as cotton, maize, sugarcane etc. The tractor-operated sprayer can be used for spraying all the 6-7 sprays on the cotton crop if same path and direction is used during all the sprayings. This reduces crop damage under the tractor. The pest control is better in case the spraying is done by tractor- operated sprayer as compared to knapsack sprayer. There is a net increase of about 29% yield in the field sprayed by tractor-mounted sprayer as compared to that sprayed by knapsack sprayer.

Fig. 10. Tractor drawn boom Sprayer

3.2.3.4 Blower Sprayers: They are also called mist sprayers. They are used to apply pesticides in concentrated form. It requires 20-80% less water and saves labour cost substantially. Further savings can be made in the quantity of chemicals used by reducing runoff from foliage when the equipment is operated properly. These sprayers are used for spraying chemicals on fruit trees, large shade trees, vegetable and certain other crops.

Fig. 11. Tractor Drawn Blower Sprayer

3.2.3.5 Air blast Sprayer: Air blast sprayers also known as air carrier sprayers have been developed for applying spray liquid to tall trees. Air blast sprayer uses air stream to carry the droplets and utilize smaller droplet size to obtain adequate coverage with low amount of spray material per unit area. The effectiveness of sprayer depends upon its ability to disperse air in all parts of trees with spray-laden air. Deposits on leaf surfaces decrease in proportion to air velocity. Drift problems with this machine are comparable with those of aircraft spraying. Most of the large orchard-type air blast sprayers have axial flow fans with guide vanes to direct the air radially outward through a partial circumferential slot. Some sprayers have two opposite axial flow fans blowing towards each other from either side of the slot. The machine can cover one side of one row or the adjacent side of two rows. The included angle of delivery on each side is adjustable to accommodate different size of trees.

Fig. 12. Tractor Drawn Air blast Sprayer

3.2.3.6 Intra canopy sprayers

The intra-canopy sprayers are capable of applying pesticide from all three sides of the plant. The spray of chemical is done from top and both side of the canopy to have better penetration of droplets in the tall crops like cotton and pigeon pea. Provision of the air blast has also been made to allow the droplets for proper penetration of the pesticide to desired locations where insects lay their eggs or hide in other stages of their growth and give better application efficiency. A front mounted six row intra canopy spraying system for operation with self-propelled high clearance power unit with 30 hp engine has been developed at CIAE, Bhopal. The sprayer is provided with six vertical sleeves with five-nozzle arrangement for each boom, centrifugal blower having air discharge of 2m /s, liquid tank of 1000 litres capacity sufficient to cover about 2 ha area in single filling of the tank and one extra pump for filling the liquid in the tank. The arrangement for lowering and lifting has been provided for adjusting the height of spray of the chemical on the crop foliage. An arrangement has been made in the system to adjust the row-to-row spacing of the nozzle system as per the requirement of the crop. The total width of coverage can be varied from 3.6 to 7.0 m. The actual field capacity of the machine was 1.06 ha/h and 0.95 ha/h in the plots of cotton and pigeon pea crops with field efficiency of 64.2% and 59.4%, respectively.

      - Laser sensor-based herbicide applicator

A two-row tractor operated laser sensor-based herbicide applicator has been developed to spray herbicide on weeds between rows. The developed herbicide applicator has two components – laser sensor for sensing the greenness and applicator part, which has controller, solenoid valve and applicator for application of herbicide. Laser sensor acts as “eye” to the equipment, which sprays liquid only on the weed patches. Solenoid controlled valves with a two-way micro controller facilitate spraying on weed patches detected by sensors. The laser sensor is trained to sense green colour with a threshold of ± 10% (green colour for which it is trained). The row to row spacing can be adjusted from 150-450 mm to accommodate crops sown at different row to row spacing. To avoid spraying of herbicide on useful plants guards have been provided on either side of the nozzle.

- Conclusion:

With the advancement of agricultural science and development in agriculture, no doubt there is increase in production and productivity both, but simultaneously there is increase in the plant pests and diseases to a considerable extent. So it has become necessary now to use pesticide and fungicides for controlling the pests and diseases. All these chemicals are applied on plants in the form of spray and dust by using sprayers and dusters, which is commonly known as plant protection equipment. A number of chemical application equipment has been developed over the years. The efficient use of chemical using proper spraying technique helps in reducing the chemical requirement, thereby reduces the cost of cultivation to farmers. It also helps in reduction of carbon footprint created in production of the agro-chemicals and adverse impact on environment. So, it has become very necessary to have thorough and proper knowledge about construction of plant protection equipment, their selection, application method, calibration, safety precautions and maintenance. The above discussed information in the chapter will certainly fulfil these objectives.

REFERENCES:

M.M. Pandey and et all (2013). Handbook of Agricultural Engineering. Published by ICAR New Delhi.

Jain S.C and Philip Grace (2003). Farm machinery an approach. Standard Publishers, New Delhi

Ojha T.P. and A.M. Michael (2017). Principles of Agricultural Engineering, volume-I, Ninth edition, Jain brothers, New Delhi.

Sahay Jagdishwar (1977). Elements of Agricultural Engineering, second edition, Standard publishers, New Delhi.

Kepner R.A. and Roy Bainer and E.L. Berger (1987). Principles of Farm Machinery, third edition, CBS publishers and distributors pvt. Limited, New Delhi.

Agrawal K.N. and et al (2013). Crop protection - Handbook of Agricultural Engineering, Published by ICAR New Delhi.: 117-129.

Bindra D.S. (1974). Pesticide Application Equipment. Oxford and IBH Publication Co Ltd, Janmapath, New Delhi